CSS45102
Introduction to Computers and Operating Systems

# How the Robot Car Moves

# Download the project files from GitHub

- Open the ELEGOO Smart Robot Car Kit V4.0 GitHub repository.

- Select the green Code button near the top of the page.

- Select Download ZIP and save the file to your computer.

- When the download finishes, open your Downloads folder.

- Right-click the ZIP file and select Extract All, then choose a location for the extracted folder.

- Open the extracted folder to access the code and supporting files.

Important: Work from the extracted folder. Do not try to open or edit the files while they are still inside the ZIP file.

# The main idea: command, wait, change

The car is controlled by two motors: one on the left and one on the right. The program sets each motor’s direction and power, waits for a chosen amount of time, and then gives the motors a new command or stops them.

Set motor directions and speeds → wait (delay) for a measured time → set a new movement or stop

For example, moveForward(100); delay(1000); tells both wheels to move forward at power 100, then keeps that command for 1,000 milliseconds (1 second). The next instruction can change the movement. A delay() does not steer or stop the car by itself; the motor command remains in effect while the program waits.

In the motor test, the program uses this time-based approach. In the line follower, the program also reads sensors and chooses a motor command repeatedly. The car’s actual distance and turning angle can vary with battery level, floor surface, wheel grip, and the car’s starting position.

# Motor power and direction

Motor power is set with a number from 0 to 255. A larger number requests more motor power; 0 turns that motor off.

Each wheel also has a direction: forward or reverse.

Different speeds on the two wheels make the car curve. Opposite directions make it pivot or spin.

These power values are not a guaranteed speed in centimeters per second.

# Movement functions in this sketch

| Function | What it commands |
| --- | --- |
| moveForward(speed) | Both wheels forward at the same power. |
| moveReverse(speed) | Both wheels backward at the same power. |
| turnLeft(outerSpeed, innerSpeed) | Both wheels forward. The right wheel (outside wheel) uses outerSpeed; the left wheel (inside wheel) uses innerSpeed. A slower inside wheel curves the car left. Equal speeds make it go straight. |
| turnRight(outerSpeed, innerSpeed) | Both wheels forward. The left wheel (outside wheel) uses outerSpeed; the right wheel (inside wheel) uses innerSpeed. A slower inside wheel curves the car right. Equal speeds make it go straight. |
| turnLeft(speed) | Left wheel backward and right wheel forward at the same power: pivot left. |
| turnRight(speed) | Left wheel forward and right wheel backward at the same power: pivot right. |
| spinLeft(speed) | Left wheel backward and right wheel forward: spin left. |
| spinRight(speed) | Left wheel forward and right wheel backward: spin right. |
| stopMotors() | Sets both wheel powers to zero. |

This sketch defines both one-argument and two-argument versions of turnLeft and turnRight. The one-argument versions pivot; the two-argument versions curve by setting different wheel powers. spinLeft and spinRight also pivot with opposite wheel directions.

# What happens in the motor test?

After the car is powered on, the program first waits for the MODE button so it can show live line-sensor readings. After that button is released, motorTest() waits for the MODE button again. It then runs this sequence:

| Order | Command | Time | Expected movement |
| --- | --- | --- | --- |
| 1 | moveForward(100) | 1 second | Both wheels forward; car moves ahead. |
| 2 | moveReverse(100) | 1 second | Both wheels backward; car reverses. |
| 3 | turnLeft(200, 200) | 1 second | Both wheels forward at equal power; straight, not a left curve. |
| 4 | turnRight(200, 200) | 1 second | Both wheels forward at equal power; straight, not a right curve. |
| 5 | stopMotors() | 1 second | Both motor powers are zero. |
| 6 | spinRight(100) | 2 seconds | Wheels run in opposite directions; car spins right. |
| 7 | spinLeft(100) | 2 seconds | Wheels run in opposite directions; car spins left. |
| 8 | stopMotors() | No delay here | Both motor powers are set to zero. |

The durations come from the delay(...) instructions immediately after the movement commands. The source comment says the right spin lasts 3 seconds, but the actual delay is 2000 milliseconds, or 2 seconds.

After the motor test, the program waits 1 second and starts line tracking.

# How line following makes decisions

The car has three line sensors: left, middle, and right. Each sensor returns a reading from 0 to 1023. The sketch treats a reading greater than 600 as detecting the black line.

Each pass through loop() reads the sensors, prints their readings, and waits 500 milliseconds. It then chooses a motor command:

| Sensor result | Program action |
| --- | --- |
| Only the middle sensor detects the line | moveForward(130) |
| The left sensor detects it and the right sensor does not | spinLeft(140) |
| The right sensor detects it and the left sensor does not | spinRight(140) |
| All three sensors detect it | stopMotors() |
| None of the sensors detect it | moveForward(90) to continue gently ahead |
| Any other combination | moveForward(130) |

The program keeps repeating this read–decide–command cycle. The 500 ms delay is the pause in each loop; it is not a fixed duration for every movement. A new sensor reading and decision can replace the current motor command on a later loop.

# Two timing helpers

waitForStart() waits until the MODE button is pressed, then waits another 2 seconds before continuing.

stopTime(mS) puts the motor driver into standby for mS milliseconds, then takes it out of standby. This is different from stopMotors(), which sets both motor powers to zero. stopTime() is defined in this sketch but is not used by the motor test or line-following loop.

# Try it: predict, test, explain

1. What do you expect if the program runs moveForward(100); delay(2000);?

____________________________________________________________________________________________

____________________________________________________________________________________________

2. In turnLeft(200, 200), why does the car not make a clear left curve?

____________________________________________________________________________________________

____________________________________________________________________________________________

3. What is the difference between a timed motor-test command and a line-following command?

____________________________________________________________________________________________

____________________________________________________________________________________________

4. Change one motor speed or one delay in the motor test. Predict what will happen, then test your prediction with the car safely on the floor.

My change:  _____________________________________________________________________

My prediction:  _____________________________________________________________________

What happened:  _____________________________________________________________________

# Safety

Keep the car on the floor with its wheels clear of obstacles. Keep hands, hair, and loose objects away from the wheels. Be ready to switch off the car if it moves unexpectedly.

Based on Elegoo_V4_LineFollower.ino. The handout describes what the sketch commands; real movement can vary between cars and surfaces.